⚡  ENERGY FLEXIBILITY

VALORIZATION PIPELINE

Dutch Grid Capacity Data — Analytics & Prospecting Methodology

From Raw DSO Data to Ranked Industrial Postcode Zones

| Field | Detail |
| --- | --- |
| Organisation | Energy Pool |
| Domain | Energy Markets · Analytics Engineering |
| Data snapshot | Capaciteitskaart — November 2024 |
| Analysis date | July 2026 |
| Status | Active — Internal Reference |
| Audience | Technical & Non-Technical — Engineers, BDs, Executives |

# 1.  Executive Summary

This document describes the end-to-end methodology used to identify, score, and rank Dutch industrial postcode zones with the highest commercial potential for energy flexibility programs — including demand-response contracting, load-shifting, and feed-in valorization under the GOPACS and CODS market frameworks.

The Dutch electricity grid is under structural stress. Years of rapid solar and wind build-out, combined with the electrification of transport and heavy industry, have pushed many regional distribution grids past their physical capacity limits. The DSOs (Distribution System Operators — the companies that own and operate the local grid) now manage growing queues of industrial connection requests that cannot be fulfilled simply by building more hardware; grid upgrades in the Netherlands take five to ten years from planning to energisation.

This creates a direct commercial opportunity. DSOs are willing to pay to flexibility providers who can reduce consumption or inject energy at exactly the right moment, acting as a virtual grid upgrade. The pipeline described here systematically identifies which postcodes and industrial zones sit inside the most congested areas, carry the deepest queues of unmet demand, and are least likely to see that congestion resolved by infrastructure investment within the next two years.

Key output: 42 priority 4-digit postcode zones across the top 10 Tier 1 supply areas in the Netherlands — a direct input for enterprise prospecting via KvK, OpenStreetMap, and commercial B2B databases.

## Pipeline at a Glance

| Step | What happens | Output |
| --- | --- | --- |
| 0 | Load raw DSO data (5 CSV files) | 462,606 postcodes · 608 supply areas · 945 projects |
| 1 | Translate Dutch column names to English | Standardised DataFrames ready for analysis |
| 2 | Exploratory data analysis (EDA) | Null checks, color-code distributions, data quality flags |
| 3 | Derive 6 flexibility metrics | Utilisation ratios, queue intensities, stress index |
| 4 | Project relief discount | Adjusted scores penalising areas with imminent grid upgrades |
| 5 | Composite valorization score (0–1) | Single ranked score per supply area |
| 6 | Quantile-based tier assignment | 5 tiers; 61 areas in Tier 1 — Priority |
| 7 | Join scores to 54,000+ pc6 postcodes | Every postcode ranked by inherited area score |
| 8 | Collapse to pc4 zones — top 10 areas | 42 priority zones ready for enterprise search |
| 9 | Geography sanity-check | Regional labels + per-area zone counts confirmed |

# 2.  Business Context — Why Grid Congestion Creates a Market

## 2.1  How the Dutch Grid Works

The Dutch electricity network has two layers relevant to this analysis:

- TenneT — the national Transmission System Operator (TSO), responsible for the high-voltage backbone (110 kV and above). It moves bulk power between large power stations, wind farms, and regional substations.

- DSOs (Liander, Enexis, Stedin, Coteq, Rendo) — the Regional Grid Operators (RNBs in Dutch), each responsible for a geographic patch of the medium- and low-voltage distribution grid. They are the organisations whose congestion data we analyse.

Every industrial customer connects to the grid through a DSO substation. The DSO sets limits on how much power can be drawn (consumption) or injected (feed-in) at any time. When total contracted capacity exceeds the physical limit of the substation, the area is declared congested, and new connection requests are placed in a queue.

## 2.2  The Congestion Problem

Grid congestion in the Netherlands has escalated sharply since 2019, driven by three simultaneous trends:

- Renewable energy build-out: large solar farms and wind parks inject power into the MV/LV grid at the same time, overwhelming feed-in capacity in rural areas.

- Electrification of industry and transport: EV charging parks, heat pumps, and electric boilers are creating new consumption peaks in areas the grid was never designed for.

- Limited construction capacity: civil engineering contractors and electrical equipment (transformers, cables) are scarce. Even approved grid upgrades take 5–10 years to complete.

The result: hundreds of supply areas are structurally over-subscribed. DSOs cannot legally force existing customers to reduce consumption, and they cannot physically connect new customers. They are actively seeking market-based alternatives — and they have budget to pay for them.

## 2.3  The Flexibility Market — GOPACS & CODS

The Dutch flexibility market operates through two mechanisms:

| Mechanism | What it is | How we benefit |
| --- | --- | --- |
| GOPACS | Grid Operator Platform Avoiding Congestion with Sustainable energy sources. A market platform where DSOs post flexibility needs and aggregators bid. | Flexibility aggregators (like Energy Pool) can bid industrial assets into GOPACS auctions and earn dispatch fees from DSOs. |
| CODS | Congestion Management System. The operational framework by which a DSO activates a flexibility contract it has already signed. | Once a contract is signed, assets in congested areas receive activation payments whenever the DSO needs load relief — typically during peak hours. |

Market rates (2024): DSOs pay €40,000–€150,000 per MW per year for certified flexibility in congested areas. Emergency dispatch can reach €300,000/MW/year on an event basis. A single 50 MW aggregation in a Tier 1 area represents €2M–€7.5M annual revenue potential.

# 3.  Data Sources

All data is published by the Dutch DSOs and TenneT via the national Capaciteitskaart (Capacity Map) portal — a legally mandated transparency requirement. The snapshot used in this analysis is dated November 2024. All five files use semicolon (;) as field separator and a comma (,) as the decimal character, following European conventions.

## 3.1  Dataset Overview

| Variable | File | Rows | Key content |
| --- | --- | --- | --- |
| pc6 | congestie_pc6.csv | 462,606 | One row per 6-digit postcode. Congestion color codes (0–3) for consumption and generation. Links to supply area, TenneT substation, DSO, municipality, and province. |
| vg | voedingsgebieden.csv | 608 | One row per DSO supply area. Physical capacity in MW (available and required), queue depth in MW, number of unique connection requests. The primary analytical table — only this file contains actual MW figures. |
| tg | tennetgebieden.csv | 150 | Same structure as vg but at TenneT transmission level. Used for cross-referencing HV stress. |
| tc | tennetcongestie.csv | 271 | Maps TenneT substations to their congestion areas and color codes. |
| proj | projecten.csv | 945 | Planned and in-progress grid upgrade projects. One row per project with target completion date and the area being relieved. Used to apply the relief discount. |

## 3.2  The Color Code System (pc6 table)

Each postcode is assigned two color codes — one for consumption demand and one for generation (feed-in) — reflecting the congestion status visible on the public Capaciteitskaart map:

| Code | Color | Meaning for consumption | Meaning for generation |
| --- | --- | --- | --- |
| -1 | No data | DSO has not published information for this postcode | DSO has not published information for this postcode |
| 0 | Transparent | No congestion. Grid has headroom. New connections possible. | No congestion. Feed-in accepted without restriction. |
| 1 | Yellow | Grid approaching limits. New connections possible with conditions. | Feed-in possible with conditions or time restrictions. |
| 2 | Orange | Congestion present. New connections require investigation and may be refused. | Feed-in restricted. Curtailment risk for generators. |
| 3 | Red | Maximum congestion. No new connections. Existing connections may face restrictions. | Maximum congestion. Feed-in severely restricted or refused. |

In the analysis, color codes are treated as an ordinal severity scale. A postcode where both consumption and generation are red (code 3 + 3) has a combined severity of 6 — the maximum possible. A postcode with code −1 (no data) is treated as 0 (no information = no confirmed congestion) to avoid inflating severity scores.

# 4.  Analytical Methodology

The pipeline transforms raw MW figures and color codes into a single commercial prioritisation score. The grain of analysis is the supply area (voedingsgebied) because it is the only table that carries physical MW capacity data. Postcode-level results are derived by joining supply-area scores back down to pc6.

## 4.1  Step 1 — Column Renaming (Dutch → English)

All Dutch column names are translated to English equivalents at load time. This step is not cosmetic — invoeding (feed-in) and afname (consumption/withdrawal) are economically opposite concepts that must never be mixed. Consistent English names eliminate an entire class of potential errors throughout a long analytical pipeline.

| Dutch original | English name | Concept |
| --- | --- | --- |
| afname (kleurcode) | consumption_color | Grid congestion severity for demand/load (0–3 scale) |
| opwek (kleurcode) | generation_color | Grid congestion severity for feed-in / generation (0–3 scale) |
| aanwezige_transportcapaciteit_invoeding | available_feedin_capacity_mw | Physical MW the grid can receive from generators |
| aanwezige_transportcapaciteit_afname | available_consumption_capacity_mw | Physical MW the grid can deliver to consumers |
| benodigde_transportcapaciteit_invoeding | required_feedin_capacity_mw | MW committed in signed connection agreements (feed-in) |
| benodigde_transportcapaciteit_afname | required_consumption_capacity_mw | MW committed in signed connection agreements (consumption) |
| wachtrij_invoeding | feedin_queue_mw | MW waiting in queue — not yet contracted (feed-in) |
| wachtrij_afname | consumption_queue_mw | MW waiting in queue — not yet contracted (consumption) |
| unieke_verzoeken_invoeding | unique_feedin_requests | Number of distinct connection applicants (feed-in) |
| unieke_verzoeken_afname | unique_consumption_requests | Number of distinct connection applicants (consumption) |

## 4.2  Step 2 — Exploratory Data Analysis

Before any metric is derived, we inspect the raw data to understand its shape, detect nulls, and identify anomalies that could distort downstream ratios:

- Capacity columns: descriptive statistics (min, max, mean, std) to understand MW ranges and detect impossibly high or zero values.

- Color code distributions: count of postcodes at each level (−1, 0, 1, 2, 3) for both consumption and generation, to understand the overall congestion landscape.

- Red postcode share: what percentage of the 462,606 postcodes are already at maximum congestion in each direction.

Key finding from EDA: two supply areas (OS WEESP 10-9i and Medemblik Transformator 1) had available_consumption_capacity_mw reported as exactly 0 MW, producing astronomically large utilisation ratios (10,000,000,000× and 1,000,000,000×). These were caused by EPSILON handling — a deliberate small constant (1e-9) added to prevent actual division-by-zero. The resulting ratios are technically correct and meaningful: these areas have zero available headroom for consumption, which is the most extreme possible congestion state. They legitimately rank highest.

## 4.3  Step 3 — Six Derived Flexibility Metrics

All six metrics are calculated on the vg (supply areas) table. This is the only table with the MW data needed for ratio computation. Postcode-level color codes are too coarse for ratio math but useful for local confirmation later.

### Metric 1 — Capacity Utilisation Ratios

Formula: utilisation = required_capacity_MW ÷ available_capacity_MW

Calculated separately for feed-in and consumption. This is the single most important congestion signal in the dataset.

| Ratio value | Grid status | Commercial interpretation |
| --- | --- | --- |
| > 1.0 | Over-subscribed | DSOs have committed more capacity in contracts than physically exists. Structural congestion. Highest willingness to pay for flexibility. |
| = 1.0 | At limit | Exactly at breaking point. Any new connection causes congestion. |
| < 1.0 | Headroom available | Grid can still absorb demand or generation. Flexibility less urgent. |

Real result from our data: of the 608 supply areas, the top consumption utilisation ratios reached extreme values (OS WEESP: 10,000,000,000×, Medemblik: 1,000,000,000×) indicating zero available headroom. The next tier of areas (OS BERGUM CENTRALE: 1.88×, OS IJPOLDER: 1.59×, OS HAARLEMMERMEER: 1.58×) represent heavily but finitely congested areas.

### Metric 2 — Queue Intensity Ratios

Formula: queue_intensity = queue_MW ÷ available_capacity_MW

The queue tells us how many MW of connection requests are waiting in line because the grid is full. Queue intensity expresses this as a multiple of the current grid capacity.

Example from the data: VENR Venray (Enexis) had a total queue of 395 MW. Queue intensity above 1.0 means the backlog exceeds the entire existing grid capacity — it cannot be cleared by a single upgrade cycle. This area will remain commercially valuable for years.

- High queue intensity signals DSO urgency: companies waiting for connections are losing revenue; DSOs face regulatory scrutiny and fines.

- It also predicts market persistence: a large queue refills even after partial upgrades, meaning multi-year flexibility contracts remain viable.

### Metric 3 — Absolute Capacity Gap (MW)

Formula: gap_MW = required_capacity_MW − available_capacity_MW

The utilisation ratio tells you proportional stress; the absolute gap tells you market size. A utilisation ratio of 3.0 on a 5 MW grid produces a 10 MW gap — a niche opportunity. The same ratio on a 200 MW grid produces 400 MW of gap — a major market. Effective prospecting requires both signals.

Notable real values: MZ Maarheeze (Enexis) — 188.6 MW queue; VENR Venray (Enexis) — 395.5 MW queue; OS ULFT 20-3i (Liander) — 105.5 MW queue. These represent substantial multi-year flexibility market volumes.

### Metric 4 — Grid Stress Index (Combined, Log-Scaled)

Formula: stress = log(1 + (feedin_util + consumption_util) / 2)

A single compound stress number combining both directions. The log₁₊ₓ transformation is deliberate: raw utilisation ratios in this dataset range from near 0 to 10 billion. Without compression, two extreme outlier areas would capture 100% of the scoring signal. log1p preserves rank order while dampening the tail — an area at 50× utilisation is more stressed than 10×, but commercial value does not scale linearly with the ratio.

### Metric 5 — Total Queue MW (Market Size Proxy)

Formula: total_queue_mw = feedin_queue_mw + consumption_queue_mw

The raw sum of feed-in and consumption queue depth in MW. Used as the absolute market size anchor — regardless of ratios, an area with a 400 MW total queue represents a meaningfully larger commercial opportunity than one with 8 MW.

### Metric 6 — Request Diversity Index

Formula: diversity = (unique_feedin_requests + unique_consumption_requests) ÷ total_queue_mw

Divides the number of distinct connection applicants by total queue MW. A high value means many small players (aggregation opportunity — bundle small assets into a portfolio). A low value means few large industrial players (direct B2B contract opportunity). This metric informs commercial model rather than territory ranking.

## 4.4  Step 4 — Project Relief Discount

The projecten table (945 rows) lists every planned grid upgrade by area and expected completion date. A congested area where a 500 MW substation expansion breaks ground next quarter is a poor long-term flexibility target — the congestion window is short and the DSO has little incentive to sign a 3-year contract when their engineers will resolve the bottleneck in 18 months.

| Parameter | Value | Rationale |
| --- | --- | --- |
| Near-term horizon | 24 months from analysis date | Projects completing within 2 years are 'imminent relief' — the flexibility window is likely shorter than a standard contract term. |
| Discount per project | −15% multiplicative | One project typically addresses one bottleneck. Three projects = −45% discount, reflecting that congestion is actively being resolved on multiple fronts. |
| Minimum floor | 0.10 (10%) | Even heavily-relieved areas retain residual value. Construction periods create temporary re-congestion; transition-phase contracts remain viable. |
| Maximum discount | 90% (floor = 0.10) | Clamped at 6+ near-term projects. We never score an area at zero. |

Real finding: the areas with the most near-term projects included Noord-Brabant (8 near-term projects), Noord-Holland (8), CM37 (8), and CM22 (8). These heavily-invested areas were appropriately discounted even if their raw congestion ratios were high, reflecting that DSOs are already committing capital to solve the problem.

Important nuance: discounted areas are not excluded — they are deprioritised. An area with near_term_project_count = 2 may still warrant a short-term pilot contract (6–12 months). near_term_project_count = 0 supports a full 3–5 year contract proposition.

## 4.5  Step 5 — Composite Valorization Score

The composite score aggregates all five most predictive metrics into a single 0–1 number per supply area, then applies the relief discount.

Formula: score = (Σ wᵢ × minmax_normalise(xᵢ)) × relief_discount

All components are min-max normalised to [0, 1] before weighting. This ensures each weight truly controls its component's contribution independent of original units (MW vs. dimensionless ratios). Without normalisation, a component measured in hundreds of MW would dominate one measured as a ratio between 0 and 2.

| Component | Weight | Why this weight |
| --- | --- | --- |
| grid_stress_index | 30% | The gating criterion. Zero stress = zero flexibility market, regardless of other signals. Highest weight because it captures both feed-in and consumption simultaneously on a log scale. |
| consumption_utilisation_ratio | 25% | Weighted above feed-in because EPEX day-ahead price spikes are demand-driven (evening peaks 18:00–20:00 CET). Demand-side flexibility commands premium fees in these windows. |
| feedin_utilisation_ratio | 15% | Feed-in curtailment contracts exist and pay well, but at lower margins than demand-response. Generators pay less than industrial consumers for flexibility. |
| feedin_queue_intensity | 15% | Deep feed-in queue signals DSOs have already committed budget to finding market-based alternatives (not just building infrastructure). |
| consumption_queue_intensity | 15% | Deep consumption queue = industrial players actively seeking connection = high intent and high willingness to sign flexibility contracts as a bridge solution. |

## 4.6  Step 6 — Quantile-Based Tier Assignment (Critical Fix)

The initial tier design used fixed absolute bin thresholds (0.2, 0.4, 0.6, 0.8). In production on this dataset, this produced a degenerate result: over 99% of supply areas landed in Tier 5, with trace representation in higher tiers.

Root cause: the score distribution is strongly right-skewed. In a national grid dataset, the vast majority of areas are not meaningfully congested — that is the normal, healthy state of a functioning grid. A small minority of areas are genuinely stressed. The distribution therefore clusters near zero with a thin high-value tail. Fixed bins calibrated for a uniform distribution simply never reach their upper thresholds in this context.

Fix: replace fixed bins with percentile-based (quantile) cuts computed from the observed score distribution. This guarantees a meaningful spread across all tiers regardless of distributional shape:

| Tier | Percentile boundary | Guaranteed share | Commercial meaning |
| --- | --- | --- | --- |
| Tier 1 — Priority | ≥ p90 (top 10%) | ~61 areas | Immediate targets. Highest stress, deepest queues, no imminent relief. Pursue within 30 days. |
| Tier 2 — High | p75 – p90 | ~15% of areas | Strong secondary targets. Approach after Tier 1 pipeline is established. |
| Tier 3 — Moderate | p50 – p75 | ~25% of areas | Worth monitoring. Congestion approaching actionable levels. Re-check at next quarterly refresh. |
| Tier 4 — Low | p25 – p50 | ~25% of areas | Low urgency. No action needed currently. |
| Tier 5 — Minimal | < p25 (bottom 25%) | ~25% of areas | No congestion signal. Not actionable. |

Actual result after the fix: 61 supply areas qualified as Tier 1 Priority. Tier boundaries shift at each data refresh — this is correct behaviour. Tiers express relative priority within the current snapshot, which is what matters for commercial decision-making.

# 5.  Postcode-Level Analysis

## 5.1  Joining Supply-Area Scores to 462,606 pc6 Postcodes

Supply-area scores are calculated at the voedingsgebied grain. Because each 6-digit postcode belongs to exactly one supply area (via supply_area_id), all area-level metrics are joined directly to the pc6 table. Every postcode inherits the score of its parent area.

A local severity tie-breaker is then computed: postcode_congestion_severity sums the two color codes (after clipping −1 to 0), yielding a 0–6 scale. Within the same supply area, postcodes with more severe local coloring rank higher. This means a red/red postcode (severity 6) always ranks above a transparent/transparent postcode (severity 0) within the same area, even though both inherit the same valorization score.

| Severity score | Color combination | What it signals |
| --- | --- | --- |
| 6 | Red (3) + Red (3) | Maximum congestion in both directions. The specific street-block is at the epicentre of the problem. |
| 5 | Red + Orange | Critical in one direction, severe in the other. |
| 4 | Red + Yellow or Orange + Orange | Significant stress in at least one direction. |
| 2–3 | Mixed moderate | Some local signal but area-level score is the dominant factor. |
| 0–1 | Transparent / no data | No local confirmation. Area score only. |

## 5.2  From 54,000+ Priority Postcodes to Actionable pc4 Zones

54,000 ranked 6-digit postcodes is not operationally useful. Even filtered to Tier 1 areas, the list is too long for a sales team to act on without further structure. The collapse from pc6 to pc4 (4-digit postcode zones) solves this in two ways:

- Geographic legibility: '1382' maps to a recognisable district or industrial estate; '1382AB' through '1382ZZ' are invisible subdivisions of the same block. Prospecting databases, maps, and field teams all operate on pc4.

- Database compatibility: the Dutch Chamber of Commerce (KvK) API, OpenStreetMap Overpass, Google Places, and all major B2B lead databases use pc4 as their standard location grain for company search.

### pc4 Scoring Model

After collapsing, each pc4 zone receives its own secondary score with three components:

| Component | Weight | What it captures |
| --- | --- | --- |
| mean_val_score (inherited) | 45% | The supply-area valorization score. The primary congestion driver — carries forward the six-metric composite from Step 5. |
| pct_red_consumption | 30% | Fraction of pc6 postcodes within this pc4 zone showing maximum consumption congestion (color code 3). Local confirmation that area-level stress is real at postcode grain. |
| pc6_count | 25% | Number of 6-digit postcodes in the zone. Proxy for geographic density of industrial/commercial activity. More postcodes = larger potential client pool. |

Each component is min-max normalised within the set of zones in the top-10 areas before weighting. This ensures density and congestion contribute proportionally. A zone with 5 postcodes but all red scores high on local confirmation despite low density.

# 6.  Results — What the Pipeline Produced

## 6.1  Top 10 Supply Areas (Tier 1 — Priority)

After scoring and tier assignment, 61 supply areas qualified as Tier 1 — Priority. The top 10 by valorization score are listed below with their real output values:

| Rank | Supply area | DSO | Val. score | Postcodes | Total queue MW | Mean severity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | OS WEESP 10-9i | Liander | 0.5688 | 182 | — (NaN) | 3.00 (all red) |
| 2 | Medemblik Transformator 1 | Liander | 0.4702 | 7 | — (NaN) | 5.00 (red+orange) |
| 3 | OS ULFT 20-3i | Liander | 0.2679 | 240 | 105.5 MW | 2.13 |
| 4 | MZ Maarheeze | Enexis | 0.2351 | 1,302 | 188.6 MW | 4.00 (all orange) |
| 5 | VENR Venray | Enexis | 0.1870 | 1,906 | 395.5 MW | 5.26 (mostly red) |
| 6 | ISM IJsselmuiden | Enexis | 0.1846 | 321 | 83.8 MW | 3.00 (all red cons.) |
| 7 | VO Veenoord | Enexis | 0.1697 | 675 | 32.8 MW | 6.00 (all red both) |
| 8 | ZLH Zwolle Hessenweg | Enexis | 0.1615 | 544 | 142.6 MW | 1.00 |
| 9 | Stompwijk en Leidschendam-Zuid | Stedin | 0.1610 | 131 | — (NaN) | 6.00 (all red both) |
| 10 | Waddinxveen | Stedin | 0.1586 | 720 | 8.4 MW | 3.00 |

Note on NaN queue values: OS WEESP, Medemblik, and Stompwijk show NaN for total_queue_mw. This is a data publication gap — the DSO has not disclosed queue figures for these areas, not an indication of zero queue. Their extreme utilisation ratios (0 available MW) confirm structural congestion regardless.

## 6.2  Top pc4 Prospecting Zones (42 Zones across 10 Areas)

Collapsing to 4-digit postcodes within the top 10 areas produced 42 prospecting zones (slightly below the expected 50 because some smaller areas have fewer than 5 distinct pc4 districts).

Area coverage breakdown — how many pc4 zones each supply area contributed:

| Rank | Supply area | DSO | Zones | pc4 codes | Best pc4 score | pc6 coverage |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Medemblik Transformator 1 | Liander | 2 | 1771, 1775 | 0.6973 | 7 postcodes |
| 2 | OS WEESP 10-9i | Liander | 3 | 1382, 1384, 1398 | 0.5944 | 182 postcodes |
| 3 | VO Veenoord | Enexis | 5 | 7761, 7765, 7833, 7844, 7887 | 0.5868 | 567 postcodes |
| 4 | MZ Maarheeze | Enexis | 5 | 5591, 5595, 5712, 6021, 6026 | 0.5095 | 1,018 postcodes |
| 5 | VENR Venray | Enexis | 5 | 5801, 5802, 5825, 5854, 5855 | 0.4538 | 821 postcodes |
| 6 | ISM IJsselmuiden | Enexis | 5 | 8267, 8269, 8271, 8274, 8277 | 0.3932 | 321 postcodes |
| 7 | OS ULFT 20-3i | Liander | 5 | 7051, 7077, 7078, 7081, 7122 | 0.3916 | 221 postcodes |
| 8 | Stompwijk + Leidschendam-Zuid | Stedin | 3 | 2266, 2381, 2492 | 0.2824 | 131 postcodes |
| 9 | ZLH Zwolle Hessenweg | Enexis | 5 | 7721, 7722, 8024, 8028, 8035 | 0.2755 | 490 postcodes |
| 10 | Waddinxveen | Stedin | 4 | 2741, 2742, 2743, 2803 | 0.2200 | 720 postcodes |

## 6.3  Geographic Distribution of Priority Zones

The 42 pc4 zones span six distinct geographic clusters, each with different industrial profiles and DSO relationships:

| Region | pc4 codes | Province | Dominant sector / context |
| --- | --- | --- | --- |
| Noord-Holland Noord (Medemblik) | 1771, 1775 | Noord-Holland | Agricultural horticulture (greenhouse lighting and heat). Liander-managed area with 0 MW available consumption capacity. |
| Almere / Weesp area | 1382, 1384, 1398 | Noord-Holland / Flevoland | Mixed logistics and residential. OS WEESP has zero available consumption headroom — entire area is at maximum congestion. |
| Emmen / SE Drenthe (Veenoord) | 7761, 7765, 7833, 7844, 7887 | Drenthe | Agricultural and light industrial. All postcodes red on both consumption and generation (severity 6) — the most uniform congestion in the dataset. |
| Weert / Helmond / Kempen (Maarheeze) | 5591, 5595, 5712, 6021, 6026 | Noord-Brabant / Limburg | Industrial manufacturing (ASML supply chain area near Eindhoven), chemicals. 188.6 MW queue depth. |
| Venray / North-Limburg | 5801, 5802, 5825, 5854, 5855 | Limburg | Agriculture-to-logistics transition zone. 395.5 MW queue — largest absolute queue of all top-10 areas. 87.7% of postcodes red. |
| IJsselmuiden / Lelystad / Dronten | 8267–8277 | Overijssel / Flevoland | Polderland agriculture and emerging logistics. All postcodes red for consumption. |
| Achterhoek / Doetinchem (OS ULFT) | 7051, 7077, 7078, 7081, 7122 | Gelderland | Rural industrial estates, metal fabrication. 105.5 MW queue. |
| Leidschendam / Katwijk / Gouda | 2266, 2381, 2492 | Zuid-Holland | Residential-industrial mix. All postcodes red in both directions (severity 6). |
| Zwolle area | 7721, 7722, 8024, 8028, 8035 | Overijssel | Logistics and distribution. 142.6 MW queue. |
| Waddinxveen / Zoetermeer / Gouda | 2741, 2742, 2743, 2803 | Zuid-Holland | Greenhouse horticulture and light industry. |

# 7.  Pipeline Outputs

| File | Rows / scope | Primary audience | Use case |
| --- | --- | --- | --- |
| ranked_postcodes.csv | All 462,606 pc6 postcodes | CRM team, field sales | Upload to CRM for postcode-level segmentation. Filter on opportunity_tier for targeted mail campaigns. |
| ranked_supply_areas.csv | All 608 supply areas | Business development | Regional strategy. Identifies which DSO areas to prioritise and in what order. |
| ranked_operators.csv | One row per DSO | Strategic accounts, management | Which DSO has the most Tier 1 areas and largest queue MW — determines which DSO relationship to invest in first. |
| top_pc4_prospecting_zones.csv | 42 pc4 zones | Prospecting team | Direct input for enterprise search via KvK API, OpenStreetMap Overpass, or B2B databases. Ready to query. |

## Output Column Reference — top_pc4_prospecting_zones.csv

| Column | Type | Meaning |
| --- | --- | --- |
| global_rank | int | Overall rank of this pc4 zone across all 42 zones (1 = highest priority) |
| rank_in_area | int | Rank within its supply area (1–5) |
| pc4 | string | 4-digit postcode zone e.g. '5801' |
| supply_area_name | string | Name of the parent DSO supply area |
| regional_grid_operator | string | DSO managing the area (Liander / Enexis / Stedin) |
| pc6_count | int | Number of 6-digit postcodes in this pc4 zone (density proxy) |
| mean_val_score | float | Average valorization score inherited from pc6 postcodes |
| pct_red_consumption | float | Share of pc6 postcodes at maximum consumption congestion (0.0–1.0) |
| pct_red_generation | float | Share of pc6 postcodes at maximum generation congestion (0.0–1.0) |
| pc4_score | float | Final composite pc4 score (0–1). Primary sort key. |

# 8.  Next Steps — Enterprise Prospecting

## 8.1  Target Industry Sectors

Not every business in a congested postcode is a viable flexibility candidate. Ideal targets consume electricity at MW scale, have operational flexibility (can shift load without shutting down production), and are not already under a flexibility contract. SBI codes to prioritise in the KvK query:

| SBI code | Sector | Why it qualifies for flexibility |
| --- | --- | --- |
| 35.1 | Electricity production, transmission, distribution | Direct energy sector players — often operate flexible generation assets themselves. |
| 20.x | Manufacture of chemicals and chemical products | Energy-intensive batch processes. Reactor heating/cooling can often be shifted by 30–60 minutes without quality impact. |
| 24.x | Manufacture of basic metals (aluminium, steel) | Electrolysis and furnace loads — some of the most valuable flexibility assets in NL. |
| 27.x | Manufacture of electrical equipment and components | Large-scale assembly lines with HVAC and process cooling loads. |
| 10.x / 11.x | Food and beverage manufacturing | Refrigeration, freezing, pasteurisation — classic demand-response candidates. |
| 52.1x | Warehousing and storage (cold chain logistics) | Temperature-controlled storage with inherent thermal flexibility. |
| 63.11 | Data processing and hosting (data centres) | Large cooling loads and UPS systems — well-established flexibility assets. |
| 01.3 / 01.6 | Greenhouse horticulture | High-power lighting and heating systems dominant in NL — especially relevant in 17xx and Maarheeze areas. |

## 8.2  Prospecting Data Sources

| Source | What it provides | Access |
| --- | --- | --- |
| KvK Open Data API | Company name, registered address, SBI code, employee count, founding date. Filter by pc4 + SBI codes listed above. | Free API — api.kvk.nl |
| OpenStreetMap Overpass API | Industrial landuse polygons, named facilities, power infrastructure nodes. Best for identifying physical site locations. | Free — overpass-api.de |
| Google Places API | Business name, category, operational status, reviews. Validates that a site is operational. | Paid (per query) — maps.googleapis.com |
| CBS Statline | Statistics Netherlands: energy consumption per sector and municipality. Useful for market sizing per zone. | Free — opendata.cbs.nl |
| Dun & Bradstreet / Kompass | Revenue estimates, employee count, credit rating, group structure. Best for qualifying financial capacity. | Commercial licence required |

## 8.3  Lead Qualification Criteria

Once a list of enterprises per pc4 zone is generated, apply these filters before CRM entry:

- Connection capacity: ≥ 1 MVA electricity connection (below this, aggregation economics rarely work at contract level in NL).

- Operational flexibility: the site can shift load by ≥ 30 minutes without product loss or safety risk.

- Contract availability: not already enrolled in a GOPACS/CODS flexibility contract (check GOPACS participant registry or qualify by phone).

- Decision-maker access: route to Energy Manager, Facility Director, or CFO — not operations staff for initial approach.

- Site permanence: multi-year lease or owned property — ensures contract duration aligns with the flexibility window (especially important for areas with near_term_project_count = 0).

## 8.4  Data Refresh Cadence

The DSO Capaciteitskaart is updated quarterly. The pipeline should be re-run at each release to:

- Detect newly congested areas that have crossed into Tier 1 since the previous run.

- Identify areas where relief projects have completed, shifting them down the tier ranking.

- Update queue depths — a growing queue in a known Tier 1 area strengthens the multi-year contract value proposition.

- Recalibrate quantile boundaries — tier membership is relative to the current distribution snapshot.

Recommended: create a quarterly changelog comparing current vs. previous Tier 1 lists. Newly-entered areas are high-priority outreach targets (fresh congestion = DSO actively seeking market solutions). Areas that dropped out of Tier 1 may indicate a completed project — flag existing contracts for renewal review.

# 9.  Technical Reference

## 9.1  Environment

| Component | Version / detail |
| --- | --- |
| Python | 3.11.7 |
| pandas | 2.x |
| numpy | 1.x |
| Execution environment | Jupyter Notebook / JupyterLab |
| Input data format | CSV — semicolon separator (;) — comma decimal (,) |
| Output format | CSV (ranked files) · .ipynb (reproducible notebook) |

## 9.2  File Structure

project/

├── raw_data/

│   ├── congestie_pc6.csv          # 462,606 rows

│   ├── voedingsgebieden.csv        # 608 supply areas

│   ├── tennetgebieden.csv          # 150 TenneT areas

│   ├── tennetcongestie.csv         # 271 substation codes

│   └── projecten.csv               # 945 upgrade projects

├── outputs/

│   ├── ranked_postcodes.csv        # all pc6 ranked

│   ├── ranked_supply_areas.csv     # all 608 areas ranked

│   ├── ranked_operators.csv        # DSO roll-up

│   └── top_pc4_prospecting_zones.csv  # 42 priority zones

└── flexibility_valorization_analysis.ipynb  # this notebook

## 9.3  Key Parameters and Calibration

| Parameter | Default value | Where to adjust | Effect of increasing |
| --- | --- | --- | --- |
| EPSILON | 1e-9 | Section 4 (Metric 1) | No practical effect — prevents div/zero only |
| NEAR_TERM_HORIZON | today + 24 months | Section 7 (Relief) | Longer horizon = more projects qualify as 'near-term' = more aggressive discounting |
| Discount per project | −15% | Section 7 (Relief) | Higher discount = strongly penalises areas with multiple planned upgrades |
| Relief floor | 0.10 | Section 7 (Relief) | Higher floor = less penalty for heavily-relieved areas |
| Tier quantile breaks | p25/p50/p75/p90 | Section 9 (Tiers) | Adjusting p90 → p85 increases Tier 1 pool size |
| pc4 weight: mean_val_score | 45% | Section 15 (pc4) | Higher weight = pc4 ranking purely follows area-level score |
| pc4 weight: pct_red_consumption | 30% | Section 15 (pc4) | Higher weight = zones with all-red postcodes always win |
| pc4 weight: pc6_count | 25% | Section 15 (pc4) | Higher weight = larger / denser zones always rank higher |

## 9.4  Reproducibility

The notebook is fully deterministic. All cells run top-to-bottom on the same input files produce identical outputs. The only non-deterministic element is pd.Timestamp.today() used in the project relief step. To fix the analysis date for archiving:

# Replace this line in the relief section:

NEAR_TERM_HORIZON = pd.Timestamp.today() + pd.DateOffset(years=2)

# With a fixed date:

NEAR_TERM_HORIZON = pd.Timestamp('2026-07-01') + pd.DateOffset(years=2)

# 10.  Glossary

| Term | Definition |
| --- | --- |
| DSO / RNB | Distribution System Operator / Regionaal Netbeheerder. The company managing the local medium- and low-voltage electricity distribution grid. In the Netherlands: Liander (Alliander Group), Enexis, Stedin, Coteq, Rendo. |
| TenneT / TSO | The Dutch Transmission System Operator. Manages the high-voltage national backbone (110 kV and above). Owns and operates transmission-level substations. |
| Supply area (voedingsgebied) | A geographic zone fed by a single DSO substation. The primary analytical unit in this pipeline. One supply area contains tens to thousands of individual postcodes. |
| Congestion | A state in which the total contracted power flow (consumption or feed-in) exceeds the physical capacity of the grid. In congested areas, no new connections can be offered and existing customers may face restrictions. |
| Feed-in (invoeding) | Electricity injected INTO the grid by generators (solar, wind, CHP). Feed-in congestion = too much generation for the local grid to absorb — generators face curtailment. |
| Consumption (afname) | Electricity withdrawn FROM the grid by consumers (factories, buildings, EV chargers). Consumption congestion = too much demand for the grid to deliver — customers face connection refusal or load restrictions. |
| Queue (wachtrij) | MW of connection requests that have been submitted to the DSO but not yet granted a connection agreement because the grid is full. The queue is our primary market depth signal. |
| GOPACS | Grid Operator Platform Avoiding Congestion with Sustainable energy sources. The Dutch market platform where DSOs post flexibility needs and aggregators (like Energy Pool) bid contracted assets. |
| CODS | Congestion Management System. The operational dispatch mechanism by which a DSO activates a pre-contracted flexibility asset during a congestion event. |
| pc6 | 6-digit Dutch postcode (e.g. '5831SK'). The finest geographic grain in our dataset. Each postcode covers approximately one street segment. |
| pc4 | 4-digit Dutch postcode district (e.g. '5831'). Covers a neighbourhood or industrial zone. The standard grain for B2B prospecting databases, field sales, and geographic APIs. |
| KvK | Kamer van Koophandel — Dutch Chamber of Commerce. Maintains the official commercial register with SBI codes, addresses, and company structure for all Dutch legal entities. |
| SBI code | Standaard Bedrijfsindeling — Dutch industrial classification code (equivalent to NACE/ISIC). Used to filter companies by industry sector in the KvK API. |
| Valorization | In energy markets: the process of extracting economic value from a flexible asset by participating in market mechanisms — DSO flexibility contracts, EPEX day-ahead auctions, balancing markets, or ancillary services. |
| Min-max normalisation | Scaling a numeric series to [0, 1] by subtracting the minimum and dividing by the range. Ensures all components contribute proportionally to a weighted composite score. |
| Quantile tier | A tier label assigned based on percentile rank within the observed distribution, rather than an absolute numeric threshold. Adapts automatically to the actual data shape regardless of skewness. |
| Relief discount | A multiplicative penalty applied to the valorization score of supply areas where near-term grid upgrade projects are already scheduled. Prevents targeting areas whose congestion window will close before a contract can deliver value. |
| EPEX | European Power Exchange. The primary marketplace for day-ahead and intra-day electricity trading in Northwestern Europe. Evening price peaks on EPEX (18:00–20:00 CET) drive the premium pricing for demand-side flexibility contracts. |